Dear Above Board user,

Quikbuf version 2.1, included with the latest release of the

Intel utility software, has two undocumented parameters that

may  help  you to fine tune Quikbuf to  your  hardware.  Use

these  parameters ONLY if Quikbuf is not printing at  normal

speed.

The  first  parameter  is  "MC=xxx".  MC  specifies  maximum

characters  transmitted  per clock tick, the  xxx  would  be

replaced  by  a number dependent upon the  configuration  of

your system.

Next,  the PB parameter. "PB=xxx" sets printer  busy  status

hold time. To adjust PB try values of + or - 50 to determine

which way the numbers need to go, then use + or - 16 to fine

tune.  Some  printers such as the HP LaserJet  need  smaller

values to improve performance. For these printers just try +

or - 16 for the PB value.

To adjust the MC parameter, try changing the number from the

default table for your system + or - 2 until an  improvement

in  printing  speed is seen. You can use + or -  1  to  fine

tune.

Here is an example of what your command line might look like

the first time you load Quikbuf2:

C:>QUIKBUF2 256 AT E P MC=25 PB=3500

Subsequent  adjustments of the MC and PB parameters  may  be

done without rebooting your computer and without  specifying

other command line parameters:

C:>QUIKBUF2 MC=23 PB=3450

Disregard the error 510 "can't set up more than one  buffer"

message; this is expected.

Use these MC and PB default values to determine a  practical

starting point from which to begin your adjustments.

�����������������������������������������������������Ŀ

� QUIKBUF2�    PC      �  AT & MOD30  � MOD50 & MOD60 �

�����������������������������������������������������Ĵ

�         � MC    PB   �   MC    PB   �   MC    PB    �

�         �������������������������������������������Ĵ

� Parallel� 19   2928  �   24   3776  �   26   4076   �

�         �            �              �               �

� Serial  � 20   2240  �   35   2432  �   38   2600   �

�������������������������������������������������������

These were found to be optimum values for various printers.

Your computer/ printer combination may require different values.

������������������������������������������������������Ŀ

� SERIAL   �    PC      �  AT & MOD30  � MOD50 & MOD60 �

������������������������������������������������������Ĵ

� PRINTER  � MC    PB   �   MC    PB   �   MC    PB    �

������������������������������������������������������Ĵ

� HP       �            �              �               �

� LASERJET � 20   2240  �   36   2480  �   38   2600   �

�          �            �              �               �

� EPSON    �            �              �               �

� FX-185   � 20   2240  �   35   2432  �   38   2600   �

�          �            �              �               �

� OKI      �            �              �               �

� MICROLINE�            �              �               �

� 192      � 20   2240  �   34   2432  �   38   2600   �

��������������������������������������������������������

������������������������������������������������������Ŀ

� PARALLEL �    PC      �  AT & MOD30  � MOD50 & MOD60 �

������������������������������������������������������Ĵ

� PRINTER  � MC    PB   �   MC    PB   �   MC    PB    �

������������������������������������������������������Ĵ

� HP       �            �              �               �

� LASERJET+�            �              �   26   4076   �

�          �            �              �               �

� EPSON    �            �              �               �

� RX-80    �            �   24   3776  �               �

� FX-100   � 18   2928  �   24   3776  �   26   4076   �

� FX-185   � 18   2928  �   19   2928  �               �

� FX-286   �            �              �   24   3776   �

� LQ-1500  � 19   2928  �              �   24   3776   �

�          �            �              �               �

� OKI      �            �              �               �

� MICROLINE�            �              �               �

� 92       � 19   2928  �   24   3776  �   26   4076   �

� 93       � 20   2912  �   24   3776  �               �

�          �            �              �               �

� IBM      �            �              �               �

� PROPRINER�            �              �   26   4076   �

�          �            �              �               �

� PANASONIC�            �              �               �

� KX-P1091 �            �              �   24   3776   �

�          �            �              �               �

� CI 3500  � 20   2928  �   24   3776  �               �

��������������������������������������������������������