The description of Yamaha OPL3SA Series Driver for OS2

Dec 18th 96

Yamaha Corporation

1.Supported OS and Hardware

OS : OS2 Warp Version4

Targeted Hardware:YMF701,YMF701B,YMF711/718,YMF715/719

2.Supported devices

Listed below are the devices that the driver supports.

1)Wave_Out, Wave_In

1.Supported sampling rate

11.025kHz,22.05kHz,44.1kHz

2.Format

Monaural and Stereo, 8bit unsigned PCM and 16bit signed PCM

3.compressed format

No compression format is supported.

4.Full duplex

Simultaneous Wave_Out and Wave_In is not supported.

2)Midi_Out

Two MIDI devices listed below are supported by this driver.

1.OPL3

2.MPU401 for external wavetable synthesizer

3)Midi_in

MPU401 midi data recording is not supported.

4)Mixer

This driver supports the playback volume for Master, Wave, Synthesizer(for internal FM or external synthesizer), CD audio, Mic._In, Line_In. And the recording volume for CD audio, Line_In, Mic_In and loopback are supported.

5)others

Game port driver is supported.

3.Others

1)APM

Suspend/Resume is not supported.

2)Plug and Play

Plug and Play is supported .(OS/2Warp ver4)

3)Resource

Regarding the resource that this driver uses (ex. I/O address, IRQ and DMA channel), there is a constraints resulting from the hardware configuration.

Listed below is the possible resources that the driver might use.

YMF701/701B

- I/O address base

- Wave_out/in and Mixer:530h,604h,E80h,F40h

- Midi_out(OPL3):380h,388h

- Midi_out(MPU401):330h,332,334h,300h

- Gameport:201h

- Configuration register:F86h

- Interrupt

- Wave_out/in:7,9,10,11

- Midi_in(MPU401):5,7,9,10

- DMA channnel

Wave_out/in:0,1,3

YMF711/715

- I/O address base

- Wave_out/in and Mixer:530h,604h,E80h,F40h

- Midi_out(OPL3):388h,398h,3A8h,3B8h

- Midi_out(MPU401):330h,320,310h,300h

- Gameport:201h,20Ah,20Eh

- Control register:390h,620h,910h,A10h

- Interrupt

- Wave_out/in:5,7,9,10,11

- DMA channnel

- Wave_out/in:0,1,3

4)Multi-language support

Multi-language is not supported.(Only English supported.)