The Open Data-Link Interface (ODI) Overview

A discussion on why Novell is promoting the transition to the

ODI specification:

A comparision between Novell's Dedicated IPX and ODI,

an ODI architectural overview, and

a comparison between ODI and Microsoft's NDIS.

Prepared By Novell Labs Marketing

Revision 1.4

Introduction

This paper provides an overview of ODI from several vantage points.  The

first section will compare ODI with Dedicated IPX.  The second section

contains an architectural overview of ODI and explains the different

components and how they interact.  The third section is a comparison of ODI

with Microsoft's NDIS.  The focus of this paper will be in helping to

understand the advantages of ODI and explain why ODI is Novell's

recommended specification.

Dedicated IPX drivers have been used for most NetWare installations since

NetWare version 2.0A, and still has the largest installed base of any type

of  LAN driver. As with any technology, it was created to serve a distinct

purpose -- act as a bridge between adapter boards and the network

communication protocol, connecting hardware with the network operating

system.  However, many companies have now moved to larger and more complex

systems where dedicated IPX drivers have limitations.  This growing need

for added flexibility in network communication spawned the development of

Open Data-Link Interface (ODI).

ODI is a data-link specification jointly developed by Apple Computer and

Novell and published to the networking industry in 1989.  The strategic

goal of ODI is to provide seamless network integration at the transport,

network, and data-link levels.  ODI simplifies the development of network

drivers for a wide variety of network adapters and network transport

protocol stacks.  The result is easier access to a wide variety of

networked resources without requiring multiple network connections or

additional investments in hardware and software.

Section I: Comparing Dedicated IPX and ODI

IPX: The way things were

To fully understand the improved functionality offered by ODI drivers, you

must first understand how dedicated IPX works and what it's limitations

are.

Dedicated IPX on a DOS workstation allows network clients to communicate with

NetWare servers via a single protocol.

In a typical DOS IPX installation, a client uses a LAN adapter and a

dedicated IPX driver to connect to the network, employing the IPX protocol

to communicate with the NetWare server. On the server side, another LAN

adapter receives the data, configured for the IPX protocol, and processes

the incoming information.  Communication goes back and forth between these

two entities along this single channel using only the IPX protocol.

This works well if all the server has to handle are DOS and OS/2 clients

using only the IPX protocol. But networks are getting more and more

complex, network managers now need to tie in more and more disparate client

types and protocol stacks on one server. What if you need to network DOS,

AppleTalk and UNIX? This could not be done under the old Dedicated IPX

architecture.

ODI: How it addresses the need for flexible protocol support

ODI's architecture is designed to radically simplify support for multiple

protocols on a single network. Rather than installing a separate driver and

adapter for each client type that needs to be supported on the network, ODI

consolidates the support for multiple protocols under one driver. This

means that one adapter in the server can support various clients on the

network.

ODI provides transport support so a Apple Macintosh (using the AFP

protocol) can use a NetWare server to queue and print documents and save

data files that are shareable with other types of workstations. UNIX,

OS/2, TCP/IP and other clients can be supported in a similar manner.

ODI allows a network to support many different protocols (such as IPX/SPX,

TCP/IP and AppleTalk) with ease.  ODI also allows network managers to

integrate new protocols (or enhancements to existing protocols) into

existing networks as they become available.

ODI consolidates services on the client side as well. Using the right

ODI adapter/driver combination, a  workstation can access services from a

NetWare server and a UNIX host without rebooting the client. And only one

client  card is needed to support both IPX and TCP/IP.

Thus, ODI allows for multiple types of workstations to communicate

transparently with network services through a single server.  Furthermore,

ODI lets a single protocol stack communicate with multiple LAN cards, (for

example, an IPX router) which was not previously possible with Dedicated

IPX.

ODI drivers are also much easier to install and upgrade than Dedicated IPX

drivers: under Dedicated IPX the driver had to be bound to the operating

system through NETGEN or SHGEN. If a customer wanted to upgrade the driver,

he or she had to re-install (re-gen) the operating system. This can be a

time consuming process, especially when considering that the operating

system must be re-installed exactly as it was set up before -- whoever does

the installation has to know all of the defaults and settings that were set

on the original file system, otherwise the system won't be completly

functional when it is re-intalled.

ODI alleviates this problem entirely by allowing new and updated drivers to

be loaded on the fly. Simply unload the existing driver and load the new

one, and the underlying file system remains uneffected.

ODI has many advantages over Dedicated IPX.  Because of these advantages,

OS/2 has always been ODI and DOS ODI was shipped with v3.10 and v2.2

Netware and later.  A comparison of some of the features and capabilities

between the two are listed below.

A Comparison of Dedicated IPX and ODI

Frame type support

DEDICATED IPX:

Ethernet Frame types Supported:

802.3

Ethernet II

Token Ring

ODI:

Ethernet Frame types Supported:

802.2

802.3

Ethernet II

Snap

Token Ring:

802.5

802.2 Snap

BENEFITS OF ODI:

One driver allows the customer to easily support multiple

frame types on the same network.  This opens the network up

to more products and platforms.

Protocol support

DEDICATED IPX:

Supports a single protocol stack:

IPX

Appletalk (on the server)

ODI:

Supports multiple protocol stacks simultaneously:

IPX

TCP/IP

AppleTalk

OSI

...etc...

BENEFITS OF ODI:

Allows for multiple protocol support under a single card

and driver.  This provides easier maintenance for hooking

up multiple clients to a single server and accessing

resources from host servers.

Physical board support

DEDICATED IPX:

Supports up to 4 physical boards in server

ODI:

Supports up to 255 logical boards in server or as many

physical boards as will fit in the machine

BENEFITS OF ODI:

Provides support for a larger number of LAN boards.  Allows

for balancing of network load over multiple cards.

Throughput capacity

DEDICATED IPX:

Max packet size = Up to 4k

only in powers of 2 (512, 1k, 2k, 4k)

ODI:

Max packet size = Up to 24k depending on media and board

limitations

BENEFITS OF ODI:

Substantially increases total network throughput.

Ease of maintenance

DEDICATED IPX:

Drivers may not be unloaded

ODI:

Drivers can be loaded on the fly.

BENEFITS OF ODI:

Allows you to upgrade LAN drivers with minimal interruption

to network services.  No need to re-install operating system.

Additional Features

ODI:

Also supports:

Lanalyzer for NetWare

Packet Burst

Locally administered node addresses

BENEFITS OF ODI:

ODI supports network services that Dedicated IPX currently

does not support.  Additional network services will be

compatible with ODI as they are provided.

Section II: Architectural Overview

With ODI, any LAN driver can communicate with any ODI protocol stack.  The

diagram below shows the modular structure of ODI.  The three components of

ODI technology are:

Multi-Link Interface Driver (MLID)

Link Support Layer (LSL)

Protocol Stacks

Multi-Link Interface Driver (MLID)

The Multi-Link Interface Driver (MLID) is a network interface card driver

developed to the ODI MLI specifications. The MLID controls communication

between the LAN board and the Link Support Layer.  There are two parts to

the MLID: the Media Support Module (MSM) and the Hardware-Specific Module

(HSM).  The MSM is source code provided by Novell that implements the

standard functions of LAN drivers into ODI for each of the standard media

types.  The HSM is the code written by the developer to handle the LAN

board details.

When the MLID receives a packet of data, it removes the media access

information (MAC header) and passes the packet to the Link Support Layer.

Since the media details are invisible to the LSL, this modular design

provides true media independence.

Link Support Layer (LSL)

The Link Support Layer (LSL) is the technological factor that allows a

single driver to support multiple protocols and for a protocol to use

multiple drivers. When the LSL receives a packet of data from the MLID, the

LSL acts as a switchboard by determining which protocol stack should

receive the packet.  This "traffic cop" functionality eliminates the need

to write separate drivers for each frame/protocol combination, thereby

dramatically reducing the number of drivers a developer has to write and a

customer has to install.

Protocol stacks

The protocol stack receives packets of information from the LSL and is

unaware of the media or LAN board type through which it passed.  It then

removes the protocol specific header information and passes the packets on

to other higher-layer protocols or applications.  The most popular types of

protocol stacks are IPX/SPX, TCP/IP, AppleTalk, and OSI.

The modular design of ODI is the key to its flexibility.  This modularity

is created by making sure that the protocol stacks are unaware of media

knowledge and that the MLIDs are unaware of protocol knowledge. The drivers

and the protocol stacks operate independent of each other making it

possible to add new media or new protocols to the file system easily.

Section III: Comparing NDIS and ODI

NDIS was co-developed by 3COM and Microsoft.  A  drawing is included below

to compare the conceptual architectural models of NDIS and ODI.  This will

provide a basis for understanding the differences between the two

specifications. (Note: this discussion is not meant to be an engineering-

level comparison of the two specifications, but rather provide a basic

working knowledge of the important differences between NDIS and ODI.)

There are several distinct differences between ODI and NDIS that end up

affecting the value of products written to the different specifications.

While both specifications promote schemes to support multiple protocols on

an internetwork, their different approaches to enabling this functionality

result in dramatically different customer implementations.

Novell's orientation on driver support is that it should be as invisible as

possible to the customer. There are many issues to consider when trying to

make driver support invisible to customers:

*	Technical competence: Does the driver actually do what it is

supposed to do: support multiple protocols? Does it do so

efficiently and gracefully?

*	Flexibility: Can additional protocols be supported as they are

developed? What about additional media, such as FDDI or ATM?  How

difficult is it to integrate support for these new products as they

emerge? Will products based on new media or protocols work with

products developed to the existing architectural specs?

*	Backward compatibility: If you are a customer and have invested in

products that support ODI, will future versions of ODI continue to

support these products or will you have to replace drivers? What

about NDIS?

*	Reliability: How can a customer determine if a third party product

really conforms to a spec, whether it be ODI or NDIS? What

difference does this make to a customer? How well will different

products from different vendors, written to the same spec,

interoperate?

*	Development issues: How long does it take to develop ODI drivers?

What is being done to reduce development time even further? How

does this affect driver availability to the customer? What

advantages does a "modular" approach offer over a "media-dependent"

approach? How does the ODINSUP shim (available through ODI) make

NDIS driver development practically unnecessary?

Flexibility

NDIS protocol stacks are media aware --they contain media knowledge in the

protocol stack.  Almost all NDIS stacks will recognize and support Ethernet

802.3 or TokenRing 802.5 or both.  This means that drivers for other media

such as Arcnet or ATM must shim this media to make it look like 802.3 or

802.5.  As a result NDIS drivers have been much more difficult to write to

any media other than Ethernet and TokenRing.

This is not so with ODI.  ODI was designed to transparently support any

network transport regardless of the underlying media.  For instance, ODI

will integrate FDDI, wireless technology or other media types into your

present network without the need to rewrite or change the protocol stacks.

The method NDIS employs to support multiple protocol stacks can also be an

issue, especially in cases where true multiprotocol support is needed. When

a system uses multiple protocol stacks, NDIS is set up to daisy chain the

packets from stack to stack (through the Protocol Manager) until the packet

is recognized and received.  This works well as long as the data packet

always needs to go to the first protocol stack in the chain, but if the

traffic is evenly spread across several stacks, or a new stack is added in

later, then the Protocol Manager will spend a lot of time toggling between

stacks looking for the right route for data transmission and the protocol

stacks at the front of the chain will get higher performance than those at

the back.

Microsoft suggests that the protocols should be loaded in order of network

traffic use so as to speed up the throughput.  But when a new protocol is

added to the network, it is the last protocol to get passed the packet of

information.  If this new protocol is going to be used for most of the

network transportation, then to get optimal speed under NDIS, you would be

required to reload all of the protocol stacks, making sure that the new

stack was loaded first.  In any case, the customer must have knowledge of

NDIS and tweak the NDIS system to get good performance from the system.

Again, the philosophy behind ODI is to minimize customer involvement with

the system -- set the system up so that the system takes care of all of the

transport details. Under ODI, the LSL determines which protocol stack

should receive each packet of information and the loading order is

irrelevant. A stack can be added into the system and ODI automatically

supports it.  And loading and unloading drivers with ODI is much easier

than with NDIS: NDIS driver loading is done in the config.sys file, while

ODI is a TSR and, under DOS, provides more flexibility for loading and

unloading drivers.

Reliability

Another important comparison to make between ODI and NDIS is how to gauge

the reliability of drivers written to each specification.  Currently

Microsoft provides test tools to developers so that they can test their own

drivers after they write them.  No outside testing or verification is

required for the drivers, so a customer really has no way of knowing how

well tested or reliable an NDIS driver is.

This is more risky when compared to the standards Novell has set for ODI.

Novell has established a department specifically designed to test and

certify drivers written by developers.  This is done to assure that the

drivers meet specifications and run in various configurations. In a sense,

Novell Labs has established a watermark that developers must meet in order

to call a product an ODI driver.

Furthermore, once an ODI driver is certified, Novell Services treats it as

if it were one of Novell's drivers. If a customer running NetWare

encounters a driver-related problem, Novell assumes a role in getting the

problem solved. If, however, the driver is NDIS, then Services has no

documentation to refer to solve the problem and cannot do much to help the

customer.

Considered from a customer's standpoint, this is an important distinction.

If  customers base their systems on ODI drivers, they know that the

products they are using have already demonstrated a significant level of

compatibility with NetWare and can be supported if something should go

wrong. If they go with NDIS, they are taking their own chances.  In large

part because of these reliability issues, we have heard of many large

accounts that have standardized on ODI drivers.

ODI drivers are certified by Novell Labs; other drivers do not have to meet

this requirement.

Backward compatibility

Another important distinction between the two specifications has to do with

backward compatibility.  For example, consider a customer running with NDIS

v2.0 drivers that wants to upgrade to Windows NT.  NT requires v3.0

drivers.  This presents a problem if the board vendor hasn't released a

v3.0 driver for the customer's board when he/she wants to install NT.

Ironically, because ODI supports any protocol stack, an ODI driver for the

same board can be used to connect into NT through a shim called ODINSUP.

This module allows the customer to talk to the NDIS protocol stacks

unmodified over the ODI driver. ODINSUP is discussed in more detail in the

following section.

Because of Novell's customer focus ODI provides complete backwards

compatibility, making upgrading much simpler.  Again, once an ODI driver is

installed, changes to the system, whether they be new protocol stacks, new

media, or products that support a new ODI spec have no impact on the

driver's compatibility. This dramatically reduces the customer' s support

and maintenance burden.

Development Issues

Time to market

LAN drivers used to take six to eight to develop before ODI was introduced.

Because of ODI's modular architecture, development time to market has been

significantly reduced.

As we have previously outlined, ODI drivers are comprised of two parts: the

Media Support Module (MSM) and the Hardware-Specific Module (HSM).  In the

past when developers wrote drivers they had to create code to access the

protocol, service the board, and access the media. In order to simplify

driver development, Novell now provides source code to interface with

various media (the MSM) as part of the Novell LAN Driver Development Kit.

This reduces the developer's coding burden: now they can focus on creating

the hardware-specific portions of the driver and adding any optional

functions. ODI's MSM/HSM drivers can be developed in 2-3 weeks.

Chipset developers have also helped to make driver development much simpler

than it was in the past. Major silicon developers (including National,

Intel Texas Instruments and AMD) have sample designs, software source code,

bills of materials and other materials needed to build products that

support ODI. These materials are available to developers that use their

chipsets.

This simplification of the development process benefits customers in two

ways:

*	Drivers become more standardized, easier to create, and easier to

test. Ultimately, this means that they will become less expensive

to produce, resulting in lower-priced products that are available

sooner to customers.

*	As more of the coding related to the network interfaces becomes

standardized, developers can focus their attention on building

greater functionality and performance into their products. Greater

speed to market means that bug fixes and patches that increase

performance (such as the packet burst NLM) will be integrated more

quickly and with less downtime than was previously possible.

ODINSUP

As part of Novell's commitment to be interoperable, ODI supports NDIS.

ODI's modular architecture allows users to support NDIS protocol stacks.  A

module called ODINSUP allows NDIS protocol stacks to run unmodified over

the ODI LSL and talk to an ODI LAN driver (please refer to the figure on

the following page).  Now, multi-vendor network transports like IBM's

NetBEUI, DEC's LAT or 3COM's XNS can be run over a common Data-Link

(driver) specification.  This feature provides two significant benefits:

*	It allows customers to integrate, and preserve investments in,

multi-vendor networking services.  Because ODI supports NDIS

stacks, customers can standardize on ODI drivers, knowing that

these drivers will support all of their networking needs, whether

they depend on NDIS or another network transport protocol.

*	The ability to access NDIS through ODINSUP also means that

developers can develop products to the ODI specification knowing

that these drivers will be able to support NDIS protocol stacks.

This feature significantly reduces a LAN driver developer's coding

burden.

Modular architecture of ODI allows integration of new technologies without

disrupting current services, including support for NDIS stacks and FDDI.

Summary

ODI was developed with a modular architecture that makes it truly media and

protocol independent.  ODI provides  greater flexibility in configuring the

network interface, uses resources more effectively and preserves

investments in hardware and other network resources.  With ODI, integration

of new technologies can be done easily and without interrupting current

services.  ODI drivers are certified by Novell Labs and are therefore very

reliable, and are compatible with previous versions of the ODI spec.

Driver development time has been reduced due to the source code provided by

Novell; this means better products get to customers more quickly.  ODI

drivers can talk to NDIS stacks through ODINSUP, allowing customers to

standardize on one type of drivers.  Because of these advantages, Novell

recommends products written to ODI.

How to Switch to ODI

Because of these advantages, Novell is including only ODI drivers with the

NetWare v4.0 operating system.  Older dedicated IPX drivers are being

phased out -- Novell Labs discontinued the certification of dedicated DOS

IPX drivers June 1992. There are now ODI drivers available for all popular

LAN adapters.  See the attached appendix for a listing of tested drivers.

In order to assist in upgrading DOS workstations from dedicated IPX drivers

to ODI, Novell is supplying a utility, (available in upcoming client

workstation kits) called WSUPGRD, which is intended to be used by a system

administrator to upgrade many workstations which are all similarly

configured.  This utility is intended to be executed from the system login

script.  It will scan the dedicated IPX.COM file on the workstation,

determine which driver was linked in with the SHGEN utility and what

hardware options were selected.  It will then reference an installation

file created by the system administrator to select the appropriate ODI

driver, and will install it along with the ODI Link Support Layer (LSL)

module and an appropriate NET.CFG file (all of which may be configured or

defined by the system administrator).  This utility is intended for use in

upgrading large sites where workstations with similar brands of LAN

adapters are also similar in configuration (usually under the control of a

system administrator).  For example, when the command lines in the

autoexec.bat files are similar, this utility will easily upgrade all the

similar adapters.