STANDARD MICROSYSTEMS CORPORATION

Banyan Vines

For TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  How to use EZStart to copy the Banyan Vines driver files from

the SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

Note: This driver supports all SMC TokenCard EliteSeries

ISA and Micro Channel Token-Ring adapters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

PREPARING YOUR OPERATING SYSTEM

1. Using any text editor, add the following lines to your CONFIG.SYS

file:

DEVICE=<VINES directory>\PROTMAN.DOS/i:c:\<VINES directory>

DEVICE=<VINES directory>\SMC8100.DOS

2. Make sure the SMC NDIS driver file (SMC8100.DOS) and the

PROTOCOL.INI file both reside in the <VINES directory>.

3. Run PCCOPY (a VINES utility) to copy the PC's Configuration

Software and LAN Driver from the VINES server to your

<VINES directory>.

4. Run PCCONFIG (another VINES utility) to configure the PC for

the network:

o  Select "Network Card Settings". Then press the Enter key.

o  Select "Token-Ring NDIS". Then press Enter.

o  Enter the interrupt that the adapter is using. Press Enter.

o  On the "PROTOCOL.INI bindings =" line, type the entry name

that matches the "Bindings=" statement in your PROTOCOL.INI

file.

o  Press Enter.

o  Press F10 to save your configuration.

o  Press Esc to exit from the current screen.

5. Reboot your computer.

SMC and Standard Microsystems are registered trademarks; and

TokenCard, TokenCard Elite, TokenCard Elite/A, Elite Series,

EliteView, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation.  Other products and company names are

registered trademarks or trademarks of their respective holders.