#

File: finlux.info,  Node: Top,  Prev: (emerson.info),  Next: (funai.info),  Up: (remotes.info)

******************************************************************************

FINLUX

******************************************************************************

* Menu:

* Technical info::

* TV::

#

File: finlux.info,  Node: Technical info,  Prev: Top,  Next: TV,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            10/16 bits

Carrier:                32kHz

Transmits (precode & wait 18ms) + (code & wait 105ms) + (postcode)

header: 500us + 5200us

one: 500 + 530us (pulse first)

zero: 500 + 530us (space first)

Precode:        000

Postcode:       000

#

File: finlux.info,  Node: TV,  Prev: Technical info,  Up: Top

******************************************************************************

TV

******************************************************************************

10 bits Code    Function        16 bits Code    Function

------------    --------        ------------    --------

#001    0                       #201    0

#003    1                       #203    1

#005    2                       #205    2

#007    3                       #207    3

#009    4                       #209    4

#00b    5                       #20b    5

#00d    6                       #20d    6

#00f    7                       #20f    7

#011    8                       #211    8

#013    9                       #213    9

#017    P C                     #215    -/__

#019    off                     #217    P/C

#01b    1..                     #219    off

#01d    2..                     #21d    step + (1..)

#021    treble +                #21f    step - (2..)

#023    treble -                #221    treble +

#025    balance left            #223    treble -

#027    balance right           #225    base +

#029    base +                  #227    base -

#02b    base -                  #229    balance right

#02d    volume +                #22b    balance left

#02f    volume -                #22d    volume +

#031    contrast +              #22f    volume -

#033    contrast -              #231    bright +

#035    bright +                #233    bright -

#037    bright -                #235    colour +

#039    colour +                #237    colour -

#03b    colour -                #239    contrast +

#03f    mute                    #23b    contrast -

#045    clock                   #24f    shift

#049    square + -              #251    mute

#04b    ?                       #253    norm

#04d    100                     #259    clock

#057    |><|                    #261    100

#059    I II                    #263    TTC (TxC)

#05d    S M                     #265    TTM (TxM)

#071    VT                      #267    TTN (TxN)

#073    TV                      #269    REV

#077    TxC                     #26b    HOLD

#07b    TxM                     #26d    FP

#07d    TxN                     #26f    INV

#079    square                  #271    SIZE

#273    TP

#275    STR

#277    PRG

#2ff    TV

The 16 bit codes were provided by Ingvar Marstorp (marstorp@ludd.luth.se)