SNMPGET(1) USER COMMANDS SNMPGET(1)

NAME

snmpget - communicates with a network entity using SNMP GET

Requests.

SYNOPSIS

snmpget host community variable-name [variable-name]...

DESCRIPTION

Snmpget is an SNMP application that uses the GET Request to

query for information on a network entity. One or more

fully qualified object identifiers may be given as arguments

on the command line. Each variable name is given in the

format specified in _v_a_r_i_a_b_l_e_s(5).

The _h_o_s_t specification may be either a host name (which will be resolved

in the _h_o_s_t_s file) or an

internet address specified in "dot notation"

It needs a configuration file _i_p._c_n_f which looks like this:

host    	192.9.202.13

netmask 	255.255.255.0

gateway 	192.9.202.4

The _c_o_m_m_u_n_i_t_y specifies the community name for the transac-

tion with the remote system.

For example

snmpget sun public system.sysdescr.0

system.sysUpTime.0

will retrieve the variables sysDescr.0 and sysUpTime.0:

Name: system.sysDescr.0

OCTET STRING- (ascii): Kinetics FastPath2

Name: system.sysUpTime.0

Timeticks: (2270351) 6:18:23

If the network entity has an error processing the request

packet, an error packet will be returned and a message will

be shown, helping to pinpoint in what way the request was

malformed. If there were other variables in the request,

the request will be resent without the bad variable.

Adding a "-d" to the argument list will cause the applica-

tion to dump input and output packets.

SEE ALSO

variable.txt, RFC 1155, RFC 1156, RFC 1158