Vega Deluxe Utility Disk Version 2.02

VEGA.COM        This program is for using the Vega Screen Saver and for

emulation of CGA and HGC.

INSTALL.COM     This program installs different VEGA utilites including:

VEGA DELUXE high resolution Windows Drivers, VEGABOOT

for bootable games, VEGA for using the screen save

and emulation, and VEGABIOS.SYS for VGA and extended

mode compatibility.

DIAG.COM        This program performs diagnostics on the VEGA card.

ROMDATE.COM     This program returns the date of your system ROM BIOS.

VEGABIOS.SYS    This program is used to provide VGA and extended mode

compatability when using your VEGA Deluxe.  You may use

the INSTALL program to set up to use this.

VEGABOOT.SYS    This program is used with bootable games to initiate

the emulation modes.

ESU.COM         This program will demonstrate extended text modes and

can be used to initiate these modes either from within

the program or from the command line.  Type ESU /? for

help.

640X480.DRV     These files are drivers for VEGA Deluxe Hires modes to

752X410.DRV     use with MicroSoft Windows Ver 1.04 and below. To use

these drivers run the VEGA Deluxe install program,

INSTALL.COM.

PALETPOP.COM    This program will let you alter the default colors used

by your computer.  It can be made memory resident to

popup whenever you want to change colors.

PALETPOP.HLP    This file provides on-line help while using PALETPOP.

EGADEFLT.CLR    These are color palette files that can be loaded by

RED2GRN.CLR     PALETPOP.

README.BAT      This file contains extra information about the VEGA and

VEGA Utilities that are not include in the Users Guide.

VD120X25.DRV    These files are text drivers for Lotus 1-2-3 Version 2.0

VD120X43.DRV    if you wish to use 120 columns by 43 rows or by 25 rows.

These drivers may only be used with an Enhanced Color

Display (ECD) or compatible monitor.

VD132X25.DRV    These files are text drivers for Lotus 1-2-3 Version 2.0

VD132X43.DRV    if you wish to use 132 columns by 43 rows or by 25 rows.

These drivers may only be used with a Sony Multiscan

or compatible monitor.

VD120X25.1A     Lotus 1-2-3, version 1A 120 column by 25 row text driver.

It can only be used with an Enhanced Color Disply (ECD)

or compatible monitor.

VD132X25.1A     Lotus 1-2-3, version 1A 132 column by 25 row text driver.

It can only be used with a Sony Multiscan or compatible

monitor.

GD752V20.DRV    These files are graphics drivers for Lotus 1-2-3 Version

GD640V20.DRV    2.0.  Both of these drivers require a variable frequency

monitor.

GD640V1A.DRV    These files are graphics drivers for Lotus 1-2-3 Version

GD752V1A.DRV    1A.  Both of these drivers require a variable frequency

monitor.

DU.COM          Video Seven directory listing utility.  Uses entire

screen in enhanced modes.  Type DU /? for options.

CLR.COM         Clears entire screen in enhanced modes.

SWITCH.EXE      This program will change your EGA switch settings

and put your computer in the mode you specify.

This is useful for some low resolution CGA games

and for switching between MONO and COLOR modes

on a multisync monitor.

VEGAANSI.SYS    This program may be used in place of standard ANSI.SYS

and will allow access to the full screen, where ANSI.SYS

only allows access to the first 25 lines.

DLXWNDW2.DOC    Documentation for VEGA Deluxe Windows 2.x driver

installation

VDLX640.DRV     These files are drivers for VEGA Deluxe 640x480 Hires

VDLX640.GRB     mode for MicroSoft Windows Ver 2.00.  See the DLXWNDW2.DOC

VDLX640.LGO     file for more information on their use.

DLXREL9.COM     AutoCAD Release 9 ADI driver.

DLXADI2.COM     AutoCAD v2.18 or above ADI driver.

DLXINST.COM     Install program for AutoCAD drivers to change screen

resolution, colors, and interrupt vector.

READACAD.DOC    Documentation on AutoCAD drivers and install program.