12/1/88

INSTALL.EXE and ADVINST.EXE have a help feature which may be invoked

by pressing the F1 key.

1/12/89

INSTALLATION NOTE:

Some clone machines are apparently unable to implement proper wait state

timing and hence the installation software will be unable to properly

determine what the bus speed actually is.  Users of these machines will

be prompted to enter the bus speed.

1/23/89

INSTALL.EXE / ADVINST.EXE

=========================

Both INSTALL.EXE and ADVINST.EXE now accept command line parameters to

force smart-testing of BOCARAM/AT PLUS boards to a specific frame segment

during installation.  This will accomodate users with boards in their

systems which have a BIOS which does not properly report its size.  To

override the test frame segment during installation the following parameter

should be used:

/T=XXXX

where XXXX is the frame segment (a hexadecimal number) and is one of the

following:

C000

C400

C800

CC00

D000

D400

DD00

DC00

Hence if you have for example a VGA BIOS wich occupies 20K Bytes starting

at C000, and the INSTALL program locks up or finds 0 Megabytes of RAM on

your board, then executing:

INSTALL /T=C800

will force the INSTALL program to use the C800 segment for such things as

detection of RAM on the BOCARAM/AT PLUS, bus speed, CPU speed, etc.

BRPTEST.EXE

===========

For those users wishing to perform the BRPTEST.EXE repeatedly in an

attempt to find a random memory error, a command line may be added to

insert the 'Y' response to the initial screen prompt: "Do you really wish

to perform this test (Y/N)?".  The program returns an errorlevel to the

system when it returns to DOS hence the "IF ERRORLEVEL" command for batch

files may be used to halt the repeated execution upon detecting an error.

The program returns ERRORLEVEL = 0 if no error occurred, and ERRORLEVEL = 1

when an error has occurred.  The following batch instructions will repeat

the BRPTEST until a failure is found, or the batch job is terminated by a

user CTRL-BREAK (following execution of the current loop):

REM Start of batch file

:nofail

BRPTEST /Y

if errorlevel 1 goto fail

goto nofail

:fail

pause

goto nofail

REM End of batch file

7/5/89

ADVINST.EXE

===========

Parity checking is now permitted to be disabled.  This was

necessitated by the incorrect implementation of 80286 bus timing by several

clone manufacturers.  Errors in their bus timing will cause parity errors

to be generated although no error in the memory data has occurred.  Hence

in these machines, elimination of parity checking the data is required.

No additional information available at this time.