*** Rendition V2x00 Reference Drivers – Release Notes ***

Version: 3.0 beta3, build 5112 - February 9th, 1999

Quick Summary:

BETA Release.

This driver is Windows 98 native.

Windows 95 compatible.

Multi-Mon supported.

RRedline API supported.

DirectX 6 supported.

OpenGL ICD integrated.

(Remove old standalone ICD's)

New MiniGL provided (see MiniGL directory)

New control panel (under Win98 only)

Many bug fixes and stability improvements…

RELEASE NOTES:

The Win9x February Beta drivers are a worthwhile install for most Rendition users. Many issues that have been communicated to us have been resolved. While these drivers are still BETA drivers and do contain bugs, they are a major improvement when compared to any of the previous BETA releases. This BETA release contains an "all new" Verite Control Panel. Use the new Control Panel with extreme caution. There are many new features that have yet to be fully  tested and may cause undesirable behavior if used incorrectly.

NOTE: The new control panel in this release only functions under Windows 98. The next release will support all the same functionality under Windows 95 as well.

Basic features and uses for the control panel:

_First and most important ---- USE AT YOUR OWN RISK!!!!  Be sure that you completely uninstalled your current drivers before installing these. Ideally, you should be using the latest BIOS, performance and stability may be unpredictable if you are not.

Note: Settings that are chosen on the Verite Control Panel do NOT go into effect until APPLY or OK is clicked and even then you may get a Message Box prompting you for a confirmation of the selected operation or configuration. Also, some settings will not be saved after reboot. (We are working on it).

***The one exception is the CRTC positioning controls. Those update real-time.

_Setting Refresh Rates: There is now more than one way to set the refresh rate. In the middle of the right hand side of the Verite Property sheet there is a Refresh Rate group of text boxes and radio buttons. The current refresh rate is displayed next to the label (Refresh Rate) of that group. You may choose Default, Optimal, from a list of Predefined refresh rates, or (drum roll, please) create your own using our newly implemented Custom Refresh Rate Control. (this has not been thoroughly tested but is known to basically work).

_Custom Refresh Rate Control: When one chooses the Custom radio button and clicks APPLY, whether or not a custom refresh rate has been entered, the CRTC box on the left hand side of the Control Panel will now contain "live" buttons that will allow adjustments to the position, height and width of your current display. One must be cautious when using these controls. They are for advanced users and should be approached with care. Also, these are very new to this driver set and have yet to be thoroughly tested. Use these controls at your own risk and only if you are confident that you will not harm your system in so doing.

_CRTC Position Controls: When you move the mouse cursor over the buttons in this control group, you should notice that where your cursor is pointing determines the function that can be executed and is displayed as a text string directly over the button when your cursor is pointing at it. When you have selected a button corresponding to the function you would like to exercise, hit the mouse button ONE CLICK at a time and watch the result before continuing. Make sure the setting is correct.

*** You may notice a blanking of the screen when using these. This is to be expected right now. Just wait until your display returns before continuing.

_Enabling TV-Out: Only enable TV-Out if you are in 640x480x16bpp. This applies to any TV-Out implementation. To enable TV-Out, check the TV check box, choose NTSC or PAL, Underscan or Overscan, FlickerFilter(if needed), then click on APPLY at the bottom of the Property Sheet. Not all combinations of the above settings will function. Try the settings that you would like. If it does not work, let us know.  TV out support in this driver is in a rough beta form, and is much less stable than the rest of the driver.

_Bilinear Filtering and DirectDraw settings: Testing for these settings is in progress. Whether or not they function correctly has yet to be verified.

Known Issues:

1) The BIOS is no longer the means by which the clock rate is determined or set. If your BIOS usually sets the clock rate to a rate higher than 55/111 then you should copy the provided VERITE.INI file to your Windows directory. If you are experiencing the "white dots," this should fix the problem.

2) The Verite Control Panel does NOT save settings upon reboot or restarting Windows for most settings. The Color control does.

3) The Control Panel will allow you to try enabling TV-Out in other modes besides 640x480x16bpp if you have a Chrontel encoder, however, they will not work. It will cause undesirable results.

4) In TV-Out, depending on which BIOS you are using, going to a Fullscreen DosBox will not function correctly. The TV_out display will be garbled. However, you can Alt-Enter back to the Desktop.

5) In TV-Out, OpenGL games may cause a "flashing effect."

6) The CRTC positioning controls are currently being tested. They may allow you to chose settings that are undesirable.

7) In D3D, some rendering artifacts may appear. We are currently in the process of refining the D3D driver architecture. Some anomalies are to be expected. Your feedback regarding these issues is appreciated. Remember, please, these are BETA drivers.

8) We may still have rendering problems with F22-ADF an European Air War. We are working on it.

9) Depending on the video memory on board, fullscreen DVD playback will be limited accordingly. eg. on a 4MB board, you will not be able to play full screen DVD at 720x480 resolutions on a 1024x768x16 window system.

10) If the card is playing in TV-Out mode, then Win98 will display a

Macrovision error if you attempt DVD playback. Therefore DVD playback is restricted to PC-Monitor mode only at this point.

11) Issues related to refresh rates lower than 60Hz in PAL TV-Out mode are currently being looked at.

Good news:

1) In D3D, visual artifacts with multi-pass rendering have been fixed. In other words, less garbage in Unreal and Half-Life, etc.

2) Rogue Squadron should look better.

3) Erroneous, horizontal lines should no longer appear.

4) No more locks related to faster systems.

5) Clock rate settings are no longer "locked."

6) Refresh rate support is enabled.

7) OpenGL Half-Life hang is fixed.

8) Full screen Dos Box hanging intermittently is fixed.

9) Sound stuttering should be completely fixed.

10) No more annoying prompt for a *.cat file during install.

Please let us know what you think of this driver release and what your experiences with it are, both good and bad.  Post into the Rendition discussion forums, found at:

http://www.rendition.com/forums.html or at

news://news.rendition.com/