USRobotics Courier HST Telix Modem Script Release 2.1 - 01/23/90

-------------------------------------------------------------------

(C) Copyright Andrzej Taramina 1988, 1989, 1990

The USRobotics logo, Courier, and HST are trademarks of

USRobotics, Inc.

Telix is the trademark of Colin Sampaleanu and Exis Inc.

DISCLAIMER

This script is copyrighted solely to prevent others from reaping

financial gain through its sale and/or use.  Permission has been

given to Colin Sampaleanu and the Exis Inc., to

distribute this script as part of the Telix program

package.  It may not be distributed as part of any other

computer communication hardware or software package without the

written permission of the author.  This script may be copied and

distributed freely by private individuals who wish to share it

with others as long as no specific charge is assessed for this

service other than minimal fees to cover the distribution medium

and the copying service itself.  Permission is hereby granted to

System Operators of active Bulletin Boards to post the TLX_HST

ARChive for public distribution.  Due to the easily modifiable

nature of scripts,  modified versions of the script

should bear a different name and have their origin clearly

identified.

If you like and use this script along with the Telix communications

package, please consider registering Telix with Colin/Exis if you

have not already done so.  Colin has spent many hours in developing

the best comm. program on the market (shareware or commercial) and

provides a high degree of support to his users....he is deserving of

our financial support.  This will allow for release 4.0 (in planning

at this moment) to be even better still!  Do your part to keep

the Shareware concept viable and thriving in the future!  Please

register Telix.

COMMERCIAL USES OF THE HST SCRIPT

The series of TLX_HST Telix Scripts may not be used for

marketing, demonstrations, advertising, problem diagnosis or

correction OR ANY COMMERCIAL or Business purposes by modem

manufacturers, their agents, dealers, resellers, or any company

or individual involved in the sale, marketing, or maintenance of

modems or other computing devices.  Commercial and/or business

uses of the TLX_HST script(s) requires formal licensing of this

product.  To obtain a commercial-use license for the TLX_HST

script(s), the author may be contacted at the address shown at

the end of this document or on one of the Bulletin Boards listed

at the end of the TLX_HST.1ST file included with this package.

TELIX RELEASE DEPENDENCY

This script will operate with TELIX 3.12 (and later

releases of TELIX).  Due to Telix Salt Script Language

dependencies, Release 2.1 of this script WILL NOT OPERATE under

pre-3.10 releases of Telix. It will also run under 3.11 as well.

OPERATIONAL SUMMARY

TLX_HST.SLC is intended as a configuration and setup help for the

USR HST modem, and although it facilitates much of the modem's

setup tasks, some knowledge of the HST's operation and

technology is required, as well as a basic understanding of

computer communications.

If you are using TLX_HST to set up your USR Courier HST for use

with Telix 3.12 or PCBoard 14.x, two menu options (TX and PC)

will perform all firmware configuration, leaving physical switch

changes up to the user (but correct switch settings are

displayed as part of the script's operation).

TLX_HST automatically detects the ROM release level of your modem.

If your USR HST modem is at ROM level 964 or 144x TLX_HST will allow

you access to special advanced functions on various menus.

These menu functions accesse HST features only available in

964 & 144x-level modems, such as data compression (MNP level 5),

settings for the new S-register 15, 4800/7200/9600/14400 link rate

settings, and the new modem configuration screen.  Advanced

Functions are NOT available for ROM releases prior to 964.

SETTING UP THE HST FOR TELIX OPERATION

Although TLX_HST eases the setup tasks, it can't do it all.  It is

likely that you will need to change the modem's switches from

their default factory settings to the setup recommended in the

menu's TX or PC options.  Unfortunately, TLX_HST can't quite

"reach out and touch switches"; you must do this yourself.  The

script makes you aware of this requirement.

Another very important part of the modem setup is the

configuration of Telix itself.  With release 3.1x of Telix,

this is done with the ALT-O command, which updates the

Telix configuration file. Later in this document there will be

some information to help you select the proper Telix

configuration parameters for the HST modem.

I recommend that you keep your HST manual handy while you

experiment with the script.  Bug reports or suggestions for

changes or improvements should be reported to the

Rose Media BBS in Toronto.

FRIENDLY WARNING

TLX_HST can make permanent changes to the modem's internal

configuration.  Once a setting has been changed, writing the

change to Non-Volatile RAM (NRAM) implements the change until

that setting is changed and written to NRAM again.  Changes to

NRAM are maintained even when the modem power is OFF.  If you

have a working set of modem settings and wish to experiment, do

not copy the changes to NRAM.  The next hardware reset or

power-off / power-on sequence will re-establish the last NRAM

settings.  A description of the HST's basic

internal operation is included in this document; you may wish to review it

to gain a better understanding of the modem's settings.

NRAM NOTES TO PCBOARD USERS

PCBoard software insists on having total control over modem

answer operations following RING detect.  To comply with this

requirement, TLX_HST sets the RAM value of register S0 to zero to

disable automatic call answering. This script also indicates

that switch 5 be left in the DOWN (ON) position, which disables

automatic call answering and leaves this feature entirely under

software control.

SUPPORT

I will attempt to correct problems with this script, as well as

keep it current with releases of the HST's firmware and Telix

software only as time allows.  This is a personal effort which I

am sharing, but for which I receive no remuneration (except from

commercial users of the script).  Please make no unreasonable

demands for support, especially for things that are not broken;

you got this for free and if it does not perform as you would

like it to, you have a God-given right not to use it.  You may

also chose to alter the script to meet your unique requirements.

If you do so, please do not distribute the modified version

under its original name.  You may contact me on a number of

bulletin boards, a list of which is contained towards the end of

the TLX_HST.1ST file included in this ZIP archive.

HST MODEM SETUP UNDER Telix 3.12

Setting up your modem for Telix use is quite simple.  In the Telix

configuratin (ALT_O), make sure that the following parameters are

specified:

MODEM & DIALING OPTIONS

Init String:         ATZ~~~AT&B1^M

Hang Up String:      ~~~+++~~~ATH0^M

Dial abort string:   ^M

Drop DTR to Hang Up: on

Note: Use of the ~ as delay character and ^M as carriage return

will insure proper script operation under Telix release 3.12.

TERMINAL OPTIONS

Baud:         34800

XON/XOFF:     off

CTS/RTS:      on

DSR/DTR:      off    (NOTE: if DSR is on the TLX_HST will hang!)

DIALING DIRECTORY ENTRIES

Set the baud rate, for dialing directory entries that support another

USR HST modem at the other end, to 38400 baud for the best speed.

That's it.  proper switch settings are:

1,2,6,7,10: UP

All others: DOWN

INTRODUCTION TO THE HST MODEM

This document discusses some of the unique features of the HST

modem and gives more specific information on using the HST with

Telix 3.12 for Bulletin Board operations.  It is not deeply

technical, but major differences between the HST and other, more

conventional modems are covered.

HIGH SPEED PROTOCOLS AND MODEM COMPATIBILITY

The HST is fast, which is probably why you bought it in the

first place.  Without going into a philosophical discussion,

suffice it to say that there are many 9600bps modems available

commercially, and most of these modems use totally incompatible

high-speed communications protocols.  Your HST will only

communicate at 9600bps with 9600 modems made by Hayes, Microcom,

etc. unless the modems support V.32 protocols and you own a

Dual-Standard HST.

Although a 9600bps standard exists, modems

adhering to it are few, far between, and still quite expensive.

You may not like this, but that's the way it is.  The HST will,

however, communicate at lower speed with all modems supporting

the 1200/2400 standards, including other modems incompatible at

the higher speeds.

Also, when connected to another HST, the effective

modem-to-modem speed ("link rate") may exceed the nominal

9600bps rate by 10 to 30%, depending on line conditions and the

ability of your computer to send/receive data at the higher

speed.  File transfer rates of 1100-1200 cps between two HST

modems (characters per second) are not uncommon.

With the new 1440/DS modems, you can exceed a nominal 14,400 bps

with file transferes exceeding 1700 cps if the modems at both ends

are 1440/DS's.

ERROR CORRECTION

At 9600+ bps, when communicating with another HST,  both modems

use sophisticated (and proprietary) error correction techniques

which insure that the data which was received is identical to

the data which was sent.  At lower speeds (1200/2400), the HST

relies on its built-in MNP firmware (Microcom Networking

Protocol) for error-free communications with other MNP-equipped

modems.  This effectively eliminates the need for conventional

"Software" error-correction implemented in software-only file

transfer protocols such as Xmodem, Xmodem/CRC, 1k-Xmodem (a.k.a.

Ymodem), Zmodem and many others.

To take advantage of the much faster and more efficient built-in

error correction, special file transfer protocols must be used.

These protocols do not perform any error-checking and

correction; they rely entirely on the modem to do so.  Such

protocols include Ymodem-G, Imodem, and the registered version

of Zmodem (DSZ, authored and copyrighted by Chuck Forsberg).

Remember that error-free transmissions take place ONLY if you

are connected to an HST (at 9600+ bps) or to an MNP-compatible

modem of any brand at 1200 or 2400bps.  If setup properly, the

HST will return a special "CONNECT xxxxx/ARQ" message upon

connect (where xxxxx is the connect speed)  which reflects the

fact that an error-free connection has been established with the

other modem.

FACTORS AFFECTING TRANSFER SPEED

There seems to be some widespread misunderstanding of the term

"Speed" as it applies to the HST and other "high-speed" modems.

How fast does it really go?  What is its limit?  More important,

what are the factors that limit the modem's speed?

It is all very simple, really.  One end of the modem talks to

the computer, while the other end talks to another modem.  The

speed you are really interested in is the LINK RATE, which is

the speed at which the two modems transmit data.

If the HST is setup to transmit/receive at the highest possible

link rate, the effective speed will depend on the following

physical factors:

1. The maximum speed that can be achieved by the other modem,

2. The maximum speed at which your computer can communicate with

your HST modem and

3. The maximum speed at which the OTHER computer can communicate

with ITS modem.

It is intuitively obvious that the effective modem-to-modem link

rate cannot possibly exceed the slowest of 1,2, or 3 above.

Unfortunately, you can only control item #2, the speed at which

your computer communicates with your HST modem.  This speed is

NOT determined by the modem, but by the following:

1. The type of computer and communications interface you are

using and

2. The communications software you are using.

PHONE LINE NOISE

In an HST-HST connection, extraneous noise characters will not

appear on the computer's screen.  This led to the false belief

that error-correcting modems eliminate noise.  They do not.

The modem's built-in error-correction firmware (ie, MNP) insures

that the data which is received is the same as the data that was

sent in a manner similar to software-only error-corrction

protocols such as CRC (Cyclical Redundancy Check).  Thus, the

modem themselves initiate the retransmission of blocks of data

which were affected by line noise.  Although the computer does

not see it, noise affects transmissions just as negatively and

results in a slowing down of the data transfer.  On the HST

modem, erratic blinking of the RD/SD lights may be an indication

of poor line quality.

DATA FRAME OPTIMIZATION (MNP4) AND DATA COMPRESSION (MNP5)

With release level 964+ of the HST's ROM code, USR implemented

MNP level 4 (Data Frame Optimization) and MNP Level 5 (Data

Compression).  Although 964+-level HST modems are fully

downward-compatible with earlier releases of the ROM (ie, 961,

963), these features can only be used with other 964+-level (or

more recent) HST modems.

Control of MNP level 4 operation is available via S-register 15,

a new register implemented in release 964+ of the HST ROM.  If

bit 4 (decimal value 16) is ON in this register, MNP level 4 is

DISABLED.   MNP level 4 operation is only effective at speeds

of 1200bps and above.

Data Compression (MNP level 5) may be a mixed blessing,

depending mostly on the type of data being transferred by the

HST modem.  If your HST is used primarily for BBS communications

and your file transfers are mostly ARChived files (processed by

SEA's ARC, Vern Buerg's ARCA, or Phil Katz's PKZIP), data

compression by the modem is not only redundant, but it may

actually SLOW DOWN your file transfers.  Effectively, the MNP 5

data compression algorithms will attempt to replicate the work

already done by the ARC utility and will fail to reduce the size

of the data any further (it will likely INCREASE the size of

your ARC'ed files), resulting in increased transfer times.

Thus, the most efficient transfer or compressed files on

964-level HST modems occurs when data compression is turned off

(to disdable MNP5, use the AT&K0 command).

ASCII, Text, .EXE and .COM files may benefit greatly from data

compression, yielding transfer rates in the 1400-1900cps range.

To activate automatic data compression, use the AT&K1 command.

HARDWARE LIMITATIONS

How fast your computer can communicate with the modem is

influenced by many factors.  The HST modem itself can

communicate with the computer at a maximum rate of 38,400bps.

Earlier models of the IBM PC and compatibles utilized an

8088 processor running at 4.77Mhz, transmitted data internally

over an 8-bit wide data path, and their serial interfaces (to

which the modem is attached) also used an 8-bit processor chip.

So-called "Turbo" machines which use a

dual-speed 8088 or NEC V20 microprocessor, or some of the 8086

or V30 machines can achieve significantly faster serial speeds.

The new series of AT-class 80286 and 80386 based machines can

provide much faster serial speeds.

The original serial processor chip is usually an 8250, and is

rated at 9600 (some 8250s can run satisfactorily at 19200,

however).  The newer 16450, a plug replacement for the 8250, is

nominally rated at up to 38400.

Regardless of the speed of your computer and serial interface,

what's at the other end is just as important.  Running a 20Mhz

386 system and with serial interface set at 38400bps is of no

consequence if the system you are communicating with is limited

to a 9600bps serial speed.  The SLOWEST link in the entire

communication chain sets the pace for all the rest, folks.

Also, be aware that computer networks running multi-node

bulletin board systems have many other tasks to perform besides

communicating over telephone lines.  These other processes may

interfere with reliable communications at high speeds, and the

sysops may purposely slow down the serial interface speed to

prevent possible conflicts arising from this situation.  One of

the most common problems in this environment is an aborted upload

when concurrent disk updates on the network's file server take

precedence over communications interupts and cause loss of

incoming data.

Other factors which can negatively affect high speed

transmissions are not readily apparent at lower speeds.  A slow

hard disk, for example, can drop the effective data transfer

rate by a few percentage points.  You may want to try a RAM

disk, for example, which eliminates disk access as a limiting

factor to data transfer performance.

COMMUNICATIONS SOFTWARE

How fast your serial interface operates is a function of the

serial interface itself, since it has an upper speed limit, as

well as the communication software you use, since IT physically

sets the speed of your communications port.  In fact, the

communications software you use may be your worst enemy.

While many commercial communications packages are good and

offer an outstanding array of features, few of them measure up

to Telix in terms of raw data transfer performance, especially

with an HST-class modem.  A number of communications packages

limit the maximum serial rate to 9600bps, regardless of your

computer and serial interface's inherent abilities.  Telix

release 3.12 can set you serial speed all the way up to

38,400 bps.

Furthermore, some otherwise good communications packages offer a

very limited choice of transmission protocols which cannot take

advantage of your computer and communications hardware.  Not

providing protocols such as Ymodem-G and Imodem, or an easy

interface to external protocols such as DSZ, severely limits the

effectiveness of your communications setup.  Your choice of

Telix is a good one, offering internal ZModem and many others along

with the most powerful script language around...stick with it!