%OP%DEThe Printer filter

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,12,80%%C%%H1%%H2%T h e   P r i n t e r   f i l t e r%H2%%H1%

All output sent to the ':PRT.0' device is routed via the printer filter. This

is effectively the code translation system defined by the printer editor

application; it allows the output sent to ':PRT.0' to be in a standard form

while allowing control codes peculiar to a certain type of printer to be sent

to the printer itself. Note that this is NOT the same as a more general type of

'filter' which will be discussed later.

The printer filter has its own on/off control, so rather than sending codes to

switch the printer on, one sends codes to switch the printer filter itself on

or off. Switching the filter on and off sends the printer on and off sequences,

as defined in the Printer Editor. These should always be sent at the beginning

and end of each printer sequence. If the filter is not switched on then

everything sent (other then the on-message) is ignored. The control codes

recognized by the printer filter are all prefixed by ENQ (ASCII code 5) and use

the same mechanism for specifying the number of following parameters as the

screen driver. Note that although a count of '1' is not strictly necessary it

is possible to include it and PipeDream, for example, does so.

%H2%Control codes%H2%

5, '['

5, ']'

5, '2','P',32+n

5, '2','H',32+n

5, 'S'

5, '3','$','x','y'

%H2%Toggle display modes%H2%

5, 'U'

5, 'B'

5, 'X'

5, 'I'

5, 'L'

5, 'R'

5, 'A'

5, 'E'

The toggles can be automatically reset at the next slot or the next carriage

return (CR) by setting the "Off at CR" option in the Printer Editor to Yes. By

default all the display toggles except A (alternate font) and E (user defined)

are automatically reset at a new slot or a carriage return.

The following codes, in addition to SPACE to DEL ($20 - $7F) are sent to the

printer:

The printer output can be redirected using the CLI (command line interpreter),

and other output can be redirected to the printer. The following diagram shows

where redirection occurs. The use of the CLI is detailed in the next section,

"Standard I/O and the CLI".

Application

The Printer Filter also provides character translations. This facility is used

in the default driver to generate a pound sign, which requires a sequence of

codes to make the printer change character sets. The UK Z88 computer has 9

translations and the international version have a further 28. The translations

may be set up using the PrinterEd application or by setting values directly

with the OS_SP call, see the "Miscellaneous useful routines" section.

%P0%

%CO:B,12,68%

NUL

BEL

BS

HT

LF

VT

FF

CR

output

redirected

to the

printer

.> :PRT

|

|

+---->-----

|

|

|

redirected printer output

( .= filename )

%CO:C,12,56%

Printer filter on

Printer filter off

Set page length to n lines

Microspace n units

New slot, reset highlights that would be reset on CR

Send $xy to the printer

Underline

Bold

Extended sequence

Italics

Subscript

Superscript (raised text)

Alternate font

User defined

($00)

($07)

($08)

($09)

($0A)

($0B)

($0C)

($0D)

PrinterEd

|

|

|

|

|

|

|

Printer ->-

filter

%CO:D,12,44%

PANEL

|

|

|

|

|

|

|

comms -->--

port

%CO:E,12,32%

9 pin

connector

%CO:F,12,20%%CO:G,12,8%